Global Station Papa Array Flanking Subsurface Mooring A & B Oxygen (O2) Data Quality Report

Evaluation Date: 8/3/2018

Evaluator: Bo Yang

Background

Ocean Station Papa (OSP), is located in the subarctic northeast Pacific Ocean (roughly at 145oW, 50oN), where repeat hydrographic cruises have been carried out since 1981 by Fisheries and Oceans Canada (Freeland 2007). A NOAA surface mooring has been deployed at OSP since 2007, for physical and biogeochemical measurements (Emerson et al. 2011; Cronin et al. 2015; Fassbender et al. 2016). A Special Oxygen Sensor Argo float (SOS-Argo F8397, WMO # 5903743) was deployed by University of Washington (UW) at OSP from June 2012 to June 2016. This float was designed to have an air-calibration mechanism (Bushinsky et al. 2016) for its Aanderaa 4330 optode oxygen (O2) sensor, which makes it capable of providing the O2 measurements with an accuracy better than ± 0.2 %. This float was operated at a cycle interval of 5 ~ 10 days covering depths from surface to 1800 m, with a vertical resolution of 3 ~ 5 m in the top 200 m of water column (Figure 1).

Figure 1. Upper Ocean (0-150 m) oxygen concentration measured by UW SOS-Argo Float F8397 at OSP. The white solid line indicates the mixed layer depth, which was determined using density criteria (de Boyer Montégut 2004). The white dash line indicates the depth of 30 m, at which the OOI subsurface moorings were deployed.

The Ocean Observatories Initiative (OOI) array was established in 2013 at OSP, with two subsurface moorings (GP03FLMA, GP03FLMB) and a profiler mooring (GP02HYPM). The locations of NOAA surface mooring, OOI Array, and the path of UW Argo float from 2012 to 2016 is presented in Figure 2. The NOAA surface mooring measured O2 concentration at ~ 5 m, and the two OOI subsurface moorings were deployed at the depth of ~ 30 m. Because the O2 concentration in the mixed layer is uniform, the O2 concentration from NOAA (5 m) and OOI (30 m) moorings were comparable when mixed layer depth (white dash line in Figure 1) was deeper than 30 m. Since oxygen has short residence time in the upper ocean, the horizontal gradient of O2 is negligible in this area (Garcia et al.) , which allows us to use the accurate Argo O2 measurements to evaluate oxygen data from OOI subsurface moorings (GP03FLMA, GP03FLMB) for the period of 2012 to 2016.

Figure 2. Locations of NOAA surface mooring, OOI array, and the path of UW SOS-Argo float (2012-16)

Method

To avoid the biases from diurnal cycle of internal wave and biological activity, oxygen data from NOAA surface mooring and OOI subsurface moorings were processed as daily mean. Data from SOS-Argo float was binned into a 2-week window and then interpolated to 1day (Bushinsky and Emerson 2015). As mentioned in the last section, we only compared the data from these platforms for the periods when the mixed layer depth was deeper than 30 m, to make sure the data was all within the mixed layer and thus comparable.

Data for OOI array were obtained from two subsurface moorings (GP03FLMA-RIS01-03-DOSTAD000, GP03FLMB-RIS01-03-DOSTAD000) in the form of NetCDF files. As shown in Figure 3, although the optode oxygen sensor outputs (in the unit of µmol L-1, without temperature and salinity corrections) were relatively complete, there was a large gap in the processed oxygen data (in the unit of µmol kg-1, with pressure, temperature, and salinity corrections) for 2014-15 due to the missing temperature and salinity data. Data availability report on the OOI data portal confirmed that the 30 m CTD data was indeed unavailable for both moorings in 2014-15 (Figure 4).

Figure 3. Optode output oxygen (‘estimated_oxygen_concentration’), processed oxygen (‘dissolved oxygen’), salinity (‘practical_salinity’), and temperature (‘'ctdmo_seawater_temperature’) data from two OOI subsurface moorings. Texts in the parentheses indicate the variable names in the NetCDF files.

Figure 4. A screenshot from data portal showing that the 30 m CTD data required for oxygen calculation was not available (highlighted with red boxes) for both subsurface moorings (GP03FLMA & GP03FLMB) in 2014-15

Despite the fact that the CTD data was missing, the OOI data portal could still plot both temperature-corrected and pressure-temperature-salinity corrected oxygen (Figure 5). Further investigation showed that the raw oxygen data and temperature-corrected oxygen were exactly the same (Figure 5, upper panel), indicating no temperature correction was applied. However, the raw oxygen data and the pressure-temperature-salinity corrected oxygen were not the same (Figure 5, lower panel), indicating that some sort of correction was applied (my best guess is pressure correction, since it is known that the deployment depth is around 30 m). This bug could be misleading for public who only use the online data portal GUI, and should be fixed in the future.

Figure 5. A screenshot from data portal showing that it could still plot both temperature-corrected and pressure-temperature-salinity corrected oxygen, despite the fact that the CTD data was missing

To salvage the oxygen data from OOI subsurface moorings for 2014-15, the salinity and temperature from UW SOS-Argo were used for the pressure-temperature-salinity correction. The algorithm can be found at the OOI github (https://github.com/ooici/ion-functions/blob/master/ion_functions/data/do2_functions.py) (Garcia and Gordon 1992, 1993). An example is shown with GP03FLMA data (Figure 6), the filled data (pink circles) merged smoothly with the rest of data, indicating the “data salvage” was a success. However, this process would be difficult for someone without substantial knowledge about chemical oceanography and/or coding.

Figure 6. An example of oxygen data from GP03FLMA. Red cross represents the optode output oxygen data (‘estimated_oxygen_concentration’), blue circle represents processed oxygen data (‘dissolved oxygen’), and the pink circles are the “salvaged” oxygen data processed with Argo salinity and temperature for pressure-temperature-salinity correction

Results

Because UW SOS-Argo was calibrated with air every 5 ~ 10 days throughout the 4-year deployment and thus had the best accuracy of all 4 platforms, herein the UW SOS Argo oxygen data was used as a “standard” to evaluate the performance of oxygen sensors on NOAA and OOI moorings and the result is shown in Figure 7.

Figure 7. Comparison of oxygen data from 4 platforms at OSP. The grey box indicates the period when OOI subsurface moorings had CTD data missing and the raw optode oxygen data was corrected using temperature and salinity from UW SOS-Argo float.

There are only data from 2 deployments available for the NOAA surface mooring from 2012-16. The data was smooth throughout each deployment, indicating there were not significant power-shortage or bio-fouling issues (it is a pumped system with the Anti-Foulant Devices). On the other hand, the oxygen sensors on the 2 OOI subsurface moorings had very similar behavior and the differences between them was small (Table 1). They both had smoother data at the beginning but noisy data after 8-9 months of deployment. The annotation on OOI data portal indicates there might be bio-fouling at the end of each deployment.

Table 1. Comparison between OOI moorings

| Time Period | Mean difference (GP03FLMB − GP03FLMA) ± sd (µmol kg-1) |
| --- | --- |
| 9/13/2013−4/22/2014 | −1.7 ± 1.8 |
| 7/15/2014−1/30/2015 | −2.7 ± 2.0 |

When comparing to UW SOS-Argo, oxygen data from NOAA mooring had good agreements with Argo float during the 2012-13 deployment. However, for the deployment in 2015-16 data from NOAA mooring had a very different pattern, which could not be explained at this moment. On average, GP03FLMA was 4.4 – 6.7 µmol kg-1 lower than Argo, while GP03FLMB was about 4.4 – 6.7 µmol kg-1 lower.

Table 2. Comparison of NOAA and OOI moorings with UW SOS-Argo

| Platform | Mean difference (Platform − Argo) ± sd (µmol kg-1) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 6/10/2012−6/16/2013 | 9/13/2013−4/22/2014 | 7/15/2014−1/30/2015 | 10/6/2015−5/23/2016 |
| NOAA surface mooring | 1.8 ± 2.5 | - | - | ？ |
| GP03FLMA | - | −6.7 ± 3.3 | −6.2 ± 2.6 | −4.4 ± 2.4 |
| GP03FLMB | - | −8.4 ± 3.7 | −8.9 ± 1.9 | - |

Conclusion

With the air-calibrated UW SOS-Argo oxygen measurements as the “standard”, the dissolved oxygen data from OSP NOAA surface mooring and OOI subsurface moorings were evaluated. Data from OOI moorings was noisy at the end of each deployment while the data from NOAA mooring didn’t seem to have this issue, which could be a result of different instrument setup (pumped/unpumped system, with/without anti-fouling device). The NOAA mooring had good agreement with Argo during the 2012-13 deployment with mean difference smaller than 2 µmol kg-1, but had a very different trend in 2015-16. The two OOI moorings had similar behaviors, the mean difference between them were about 2 – 3 µmol kg-1. However, the results from OOI moorings were significantly lower than Argo float, in the range of 4 – 9 µmol kg-1.

Reference

de Boyer Montégut, C. 2004. Mixed layer depth over the global ocean: An examination of profile data and a profile-based climatology. J. Geophys. Res. 109: C12003. doi:10.1029/2004JC002378

Bushinsky, S. M., and S. Emerson. 2015. Marine biological production from in situ oxygen measurements on a profiling float in the subarctic Pacific Ocean. Global Biogeochem. Cycles 29: 2050–2060.

Bushinsky, S. M., S. R. Emerson, S. C. Riser, and D. D. Swift. 2016. Accurate oxygen measurements on modified Argo floats using in situ air calibrations. Limnol. Oceanogr. Methods.

Cronin, M. F., N. A. Pelland, S. R. Emerson, and W. R. Crawford. 2015. Estimating diffusivity from the mixed layer heat and salt balances in the North Pacific. J. Geophys. Res. Ocean. 120: 7346–7362.

Emerson, S., C. Sabine, M. F. Cronin, R. Feely, S. E. Cullison Gray, and M. DeGrandpre. 2011. Quantifying the flux of CaCO 3 and organic carbon from the surface ocean using in situ measurements of O 2 , N 2 , pCO 2 , and pH. Global Biogeochem. Cycles 25: n/a-n/a. doi:10.1029/2010GB003924

Emerson, S., and C. Stump. 2010. Net biological oxygen production in the ocean—II: Remote in situ measurements of O 2 and N 2 in subarctic pacific surface waters. Deep Sea Res. Part I Oceanogr. Res. Pap. 57: 1255–1265.

Fassbender, A. J., C. L. Sabine, and M. F. Cronin. 2016. Net community production and calcification from 7 years of NOAA Station Papa Mooring measurements. Global Biogeochem. Cycles 30: 250–267. doi:10.1002/2015GB005205

Freeland, H. 2007. A short history of Ocean Station Papa and Line P. Prog. Oceanogr. 75: 120–125. doi:10.1016/j.pocean.2007.08.005

Garcia, H. E., and L. I. Gordon. 1992. Oxygen solubility in seawater: Better fitting equations. Limnol. Oceanogr. 37: 1307–1312. doi:10.4319/lo.1992.37.6.1307

Garcia, H. E., and L. I. Gordon. 1993. Erratum: Oxygen Solubility in Seawater: Better Fitting Equations. Limnol. Ocean. 38: 656.

Garcia, H. E., R. A. Locarnini, T. P. Boyer, J. I. Antonov, O. K. Baranova, M. M. Zweng, J. R. Reagan, and D. R. Johnson. World Ocean Atlas 2013, Volume 3: Dissolved Oxygen, Apparent Oxygen Utilization, and Oxygen Saturation. NOAA Atlas NESDIS 75 27.